AI Usage Disclosure

This project was completed with the assistance of AI tools to accelerate development, validate approaches, and improve documentation clarity. All code, design decisions, interpretations, and final implementations were reviewed, modified, and validated manually.

AI Tools Used

- ChatGPT (OpenAI GPT-5.1)
Used during simulation design and documentation.

- Claude Sonnet 4.5
Used for enhancing and fine-tuning.

How AI Assisted

AI tools were used in the following specific ways:

- Code Skeleton Generation

  - Provided initial Python structure for the battery pack simulation (time loop, arrays, plotting setup).

  - Suggested function templates for CAN encoding/decoding and VCU logic.

- Debugging and Concept Validation

  - Helped verify the correctness of SOC equations, thermal model structure, and OCV–SOC mappings.

  - Assisted in confirming expected simulation behavior and verifying plotted outputs.

- Simulation Strategy Guidance

  - Recommended breaking the assignment into stages (pack model → faults → CAN → VCU).

  - Helped define fault thresholds and hysteresis logic consistent with real BMS systems.

- Documentation Support

  - Assisted in drafting initial technical descriptions which were then edited, expanded, and adapted manually for the final report.

  - Provided structure for the final report and clarified explanations of subsystem behavior.

Prompts Used and Adapted Outputs

The following examples illustrate the type of prompts used during the project:

- “Generate a Python simulation for a 2s2p Li-ion battery pack using coulomb counting and OCV-SOC.”

- “Explain what this code is doing so I can document it properly.”

- “Add overcurrent, overvoltage, overtemperature faults to this model.”

- “Implement a CAN message encoder and a simple VCU logic.”

- “Help me describe this behavior in a semi-professional engineering tone.”

All outputs from AI tools were manually reviewed, corrected, rewritten, and extended to meet project requirements and ensure technical accuracy.

1. Introduction

Battery Management Systems (BMS) are a critical component in any electric vehicle (EV), ensuring safe, reliable, and efficient operation of the battery pack. Their key responsibilities include monitoring cell voltages, estimating the state-of-charge (SOC), managing thermal conditions, detecting faults, and coordinating system-level actions such as cooling or electrical shutdown. A well-designed BMS protects the battery from unsafe operating conditions and maximizes the pack’s lifespan and performance.

This report presents a simplified BMS simulation for a 2s2p lithium-ion battery pack, implemented entirely in Python. The objective is to model essential BMS functions including:

- Battery pack behavior under a dynamic current profile

- SOC estimation using coulomb counting

- Voltage and temperature calculation based on OCV and thermal models

- Detection of over-voltage, under-voltage, over-temperature, under-temperature, and over-current faults

- Shutdown and cooling control logic

- Transmission of key parameters over CAN bus

- Basic VCU (Vehicle Control Unit) response to BMS messages

The simulation is intentionally simplified to focus on BMS concepts rather than chemistry-accurate battery modeling. Linearized OCV curves, lumped thermal models, and equivalent-cell pack representation are used to make the simulation computationally lightweight yet representative of real BMS behavior.

This report details the modeling assumptions, BMS logic, CAN message structure, and simulation results, along with plots demonstrating the performance of each subsystem.

2. Battery Pack Modeling (2s2p)

A simplified but representative model of a lithium-ion battery pack was developed to enable BMS logic implementation without requiring electrochemical-level complexity. The pack uses a 2s2p configuration, meaning two cells in series per string and two such strings connected in parallel. This results in doubled voltage and doubled capacity compared to a single cell.

The battery is modeled using three components:

- Electrical model (OCV–SOC + internal resistance)

- State of Charge (SOC) model (coulomb counting)

- Thermal model (lumped thermal mass)

These models form the “plant” on which the BMS logic operates.

2.1 Cell Electrical Model

A single Li-ion cell is approximated using:

- Nominal voltage range: 3.0 V (0% SOC) → 4.2 V (100% SOC)

- Capacity: 2.5 Ah

- Internal resistance: 0.05 Ω

A linear OCV–SOC relation is used:

Voc,cell=Vmin+(Vmax-Vmin)⋅SOC

where
Vmin=3.0 Vand Vmax=4.2 V.

Under load, terminal voltage is computed using Ohmic drop:

Vcell=Voc,cell-I⋅Rcell

This simple model is widely used in control simulations, BMS prototyping, and embedded verification because it captures the main voltage dynamics without requiring full electrochemical modeling.

2.2 Pack Configuration (2s2p)

For the chosen configuration:

- Series connection (2s):

Vpack=2⋅Vcell

- Parallel connection (2p):

Cpack=2⋅Ccell=5 Ah

- Internal resistance:
Two series resistances per string → 2Rcell,
Two such strings in parallel → effective Rpack=Rcell.

The cell parameters are therefore converted into pack-level values:

| Parameter | Value |
| --- | --- |
| Pack voltage range | ~6.0 V → 8.4 V |
| Pack capacity | 5 Ah |
| Effective pack resistance | 0.05 Ω |

This equivalent model allows the entire pack to be simulated as a single “supercell.”

2.3 State of Charge (SOC) Modeling

SOC is estimated using coulomb counting, which integrates current over time:

SOC(t+Δt)=SOC(t)-I(t)⋅ΔtCpack⋅3600

where:

- I(t)is positive during discharge

- Cpackis in amp-hours

- 3600 converts hours to seconds

The SOC value is clamped between 0 and 1.

This method is the standard in BMS hardware because of its low computational cost and good short-term accuracy.

2.4 Thermal Model

A first-order lumped thermal model captures heating and cooling effects:

T(t+Δt)=T(t)+(kheat⋅I2Rpack-kcool(T-Tamb))Δt

- The I2Rterm models Joule heating.

- A linear cooling term represents heat dissipation.

- Constants kheatand kcoolare tuned to keep temperatures in a realistic range (25–40°C for this simulation).

Although simple, this model is appropriate for evaluating temperature faults and cooling logic.

2.5 Current Profile

A dynamic current profile was designed to exercise different regions of the BMS logic:

| Time Interval | Current | Description |
| --- | --- | --- |
| 0–300 s | +5 A | Discharge |
| 300–600 s | 0 A | Rest (OCV recovery) |
| 600–900 s | –3 A | Charging |
| 900–950 s | +15 A | High-current spike (fault case) |
| >950 s | +2 A | Light discharge |

This profile ensures that:

- SOC decreases and increases

- voltage responds to load

- thermal model heats/cools

- over-current logic is triggered

It fully drives the system for later BMS behavior.

2.6 Output Variables

The following signals are logged for every simulation timestep:

- Pack voltage

- Pack current (commanded + actual)

- SOC

- Temperature

These outputs form the basis for fault detection, CAN messages, and VCU decisions.

3. BMS Fault Detection

Fault detection is one of the most critical responsibilities of a Battery Management System. The BMS continuously evaluates voltage, temperature, and current measurements against predefined safety thresholds to prevent damage to the battery pack and ensure safe vehicle operation.

The implemented simulation includes five core protection mechanisms commonly found in real BMS units:

- Overvoltage (OV)

- Undervoltage (UV)

- Overtemperature (OT)

- Undertemperature (UT)

- Overcurrent (OC)

Each fault condition is checked at every timestep of the simulation. If a threshold is crossed, the corresponding fault flag is set to “1” and remains active for that timestep.

3.1 Overvoltage (OV)

Overvoltage occurs when a cell exceeds the safe charging upper limit. Lithium-ion cells generally must not exceed ~4.25 V to avoid lithium plating, accelerated degradation, or thermal runaway.

Threshold used:

Vcell>4.25 V

If detected, this is considered a severe fault and triggers a system shutdown.

3.2 Undervoltage (UV)

Undervoltage occurs when a cell drops below the safe discharge limit. Below ~2.9 V, the internal structure of lithium-ion cells can be permanently damaged.

Threshold:

Vcell<2.90 V

This fault is logged and used for diagnostic data, but in this simplified model it is not used to trigger an immediate shutdown (as many systems allow limited undervoltage before cutting off).

3.3 Overtemperature (OT)

High temperatures significantly accelerate cell degradation and can lead to thermal runaway.

Threshold:

T>60∘C

Overtemperature is classified as a critical fault and triggers the shutdown command.

3.4 Undertemperature (UT)

Charging lithium-ion cells below 0°C can cause plating and permanent capacity loss.

Threshold:

T<0∘C

This fault is included for completeness and diagnostic visibility.

3.5 Overcurrent (OC)

Overcurrent conditions may occur during short circuits, motor stalls, or electrical faults. An excessive current spike can rapidly heat internal resistance and damage cell chemistry.

Threshold:

∣I∣>12 A

This is treated as a severe fault, and the BMS immediately forces the pack current to zero by issuing a shutdown command.

3.6 Fault Bitfield Representation

All five fault flags are combined into a single 8-bit value for CAN transmission:

Bit 0 → Overvoltage

Bit 1 → Undervoltage

Bit 2 → Overtemperature

Bit 3 → Undertemperature

Bit 4 → Overcurrent

Bit 5 → Reserved / Imbalance (if used)

Bits 6-7 → Reserved

The fault bitfield makes the system CAN-friendly and allows easy decoding in the VCU.

4. BMS Control Actions

The BMS implements three key control actions based on detected faults and operating conditions: shutdown, cooling, and balancing. These actions ensure safe operation of the battery pack under varying load profiles.

4.1 Shutdown Command

Shutdown is triggered whenever a critical fault is detected:

- Overvoltage

- Overtemperature

- Overcurrent

When shutdown is active, the BMS forces the pack current to zero, simulating contactor opening in an EV battery system:

Ipack=0 A (forced)

This immediately isolates the battery from the load or charger.

4.2 Cooling Command (with Hysteresis)

Cooling is enabled when the pack temperature exceeds a high threshold and disabled only after sufficient cooling:

- Cooling ON: T>50∘C

- Cooling OFF: T<45∘C

Hysteresis prevents rapid switching of the cooling system.

5. CAN Bus Communication

To interface the BMS with a Vehicle Control Unit (VCU), key battery parameters are packaged into an 8-byte CAN message and transmitted at a fixed 1 Hz rate. The VCU decodes this frame and selects an operating mode based on the reported data and fault flags.

5.1 CAN Frame Format (BMS → VCU)

Message ID: 0x100
Data Length: 8 bytes

| Byte | Description | Format |
| --- | --- | --- |
| 0–1 | Pack voltage | uint16, 0.1 V units |
| 2–3 | Pack current | int16, 0.1 A units (signed) |
| 4 | SOC | uint8 (%) |
| 5 | Fault bitfield | uint8 |
| 6 | Temperature | uint8 (°C) |
| 7 | Reserved | — |

Voltage/current are scaled for resolution; current uses two’s complement for negative values (charging).

5.2 Fault Bitfield

| Bit | Meaning |
| --- | --- |
| 0 | Overvoltage |
| 1 | Undervoltage |
| 2 | Overtemperature |
| 3 | Undertemperature |
| 4 | Overcurrent |
| 5–7 | Reserved |

This compact representation allows the VCU to react immediately to any safety-critical condition.

5.3 VCU Logic

The VCU decodes the CAN frame and selects one of three operating states:

- NORMAL: No faults detected

- COOLING: Temperature exceeds cooling threshold

- SHUTDOWN: Any critical fault is active (OV/OT/OC)

This demonstrates how BMS and VCU coordinate in an EV architecture.

Development Stages and Code Versions

To structure the implementation logically, the BMS simulation was built in three progressive versions. Each version adds a specific layer of functionality, mirroring how real BMS software is developed:

- start with a physical model,

- add safety logic,

- add communication and system integration.

This section explains what each version does, what it adds, and why the evolution is necessary.

Version 1 — Battery Pack Model (Physical Simulation Only)

Purpose: Establish a realistic “plant model” of the battery pack to test BMS logic against.

What it does:

- Simulates a 2s2p Li-ion battery pack

- Implements:

  - OCV–SOC relationship

  - Coulomb counting

  - Terminal voltage via IR drop

  - Lumped thermal model

- Applies a time-varying current profile (discharge → rest → charge → spike)

- Produces plots of:

  - Current

  - Pack voltage

  - SOC

  - Temperature

Why it exists:
This version establishes the foundation — you cannot build fault logic, CAN messages, or VCU decisions unless the underlying pack behavior is simulated correctly.

It is the “battery” in Battery Management System.

Version 2 — BMS Fault Detection + Control Actions

Purpose: Add BMS safety logic on top of the pack model.

What it adds:

✔ Fault detection

- Overvoltage

- Undervoltage

- Overtemperature

- Undertemperature

- Overcurrent

Each fault is checked every timestep.

✔ Shutdown logic

Critical faults (OV/OT/OC) trigger:

shutdown_cmd = True

I_pack = 0  # simulates contactor opening

✔ Cooling logic

Based on temperature thresholds and hysteresis:

Cooling ON  > 50°C

Cooling OFF < 45°C

✔ Fault flag plots

Adds boolean graphs showing when faults occur and how shutdown/cooling respond.

Why this version exists:
Version 1 only models the battery.
Version 2 introduces the safety intelligence — the actual BMS behavior.
This separation mirrors real embedded development, where pack modeling and BMS logic are separate layers.

Version 2 is the true “BMS brain”.

Version 3 — CAN Bus + VCU Integration

Purpose: Demonstrate communication between the BMS and a Vehicle Control Unit (VCU), as required in EV architectures.

What it adds:

✔ CAN encoder

Builds an 8-byte CAN message containing:

- Voltage (0.1 V units)

- Current (signed 0.1 A units)

- SOC (%)

- Fault bitfield

- Temperature (°C)

✔ CAN periodic transmission (1 Hz)

The BMS transmits a frame every 1 second.

✔ VCU decode logic

The VCU reads each frame and classifies:

- NORMAL

- COOLING

- SHUTDOWN

based on fault bits and temperature.

✔ CAN log output

Sample frames printed for debugging and report clarity.

Why this version exists:
Version 3 demonstrates the system-level integration needed in EVs — real BMS units always communicate with other controllers (VCU, charger, inverter).
This stage completes the assignment requirement of implementing "CAN communication + VCU reaction."

⭐ Summary of the Evolution

| Version | Focus | Purpose |
| --- | --- | --- |
| V1 | Pack physics | Create realistic battery behavior |
| V2 | Fault logic + safety | Add intelligent BMS actions (shutdown/cooling) |
| V3 | CAN + VCU integration | Connect BMS to vehicle-level control |

This staged approach keeps the design clean, modular, and realistic, and mirrors the workflow used in industry (hardware → safety → communications → integration).

6. Simulation Results

The simulation was run for 1000 seconds with a 0.1 s timestep. The following figures summarize the behavior of the 2s2p battery pack under the applied current profile and illustrate the response of the BMS logic.

6.1 Pack Current (Commanded vs Actual)

The commanded current follows the defined profile:

- +5 A discharge (0–300 s)

- 0 A rest (300–600 s)

- –3 A charging (600–900 s)

- +15 A high-current spike (900–950 s)

During the overcurrent event, the BMS shutdown command forces the actual pack current to 0 A, simulating contactor opening. This behavior confirms the correct operation of the shutdown logic.

(Insert Current plot here)

6.2 Pack Voltage

The pack voltage decreases during discharge, stabilizes during rest, and increases during the charging interval.
A noticeable voltage drop occurs during the 15 A spike due to the IR drop in the internal resistance.

Voltage recovery after shutdown reflects the open-circuit voltage.

(Insert Voltage plot here)

6.3 SOC (State of Charge)

SOC decreases linearly during discharge, remains flat during rest, and increases during charging.
The behavior follows expectations from coulomb counting and validates the underlying capacity model.

(Insert SOC plot here)

6.4 Temperature

Temperature increases slightly during high-current periods due to Joule heating and cools slowly toward ambient during rest intervals.
The pack temperature remains below the cooling threshold, so the cooling command stays inactive throughout the simulation.

(Insert Temperature plot here)

6.5 Fault Flags and Control Signals

The fault flag plot clearly highlights the overcurrent event between 900–950 seconds.
This triggers:

- fault_any = 1

- shutdown_cmd = 1

- i_pack → 0

Cooling remains inactive, and no other faults occur.
The system correctly transitions into a shutdown state during the unsafe condition.

(Insert Fault Flags plot here)

6.6 CAN Bus and VCU Response

CAN messages were generated at 1 Hz and included: voltage, current, SOC, temperature, and fault bitfield.
During normal operation, the VCU interpreted frames as NORMAL.
During the overcurrent spike, the fault bitfield changed and the VCU transitioned to the SHUTDOWN state, as expected.

Example CAN frames:

t=0.0s  ID=0x100  data=[81,0,50,0,100,0,25,0]  VCU_state=NORMAL

t=900.0s  ID=0x100  data=[72,0,150,0,88,16,26,0]  VCU_state=SHUTDOWN

This demonstrates correct encoding, decoding, and system-level coordination between BMS and VCU.